Sweep Tube Comparison

| Tube | Plate Dissipation Watts | Screen Dissipation Watts | Tranconductance Micromhos | Heater | Capacitances |  |  | RF Operation (up to 30 MHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (6.3V) Amperes | Cin pF | Cgp pF | Cout pF | Class of Service | Plate Voltage | Screen Voltage | Grid Voltage | Plate Current mA | Screen Current mA | Grid Current mA | Approx. Driving Power Watts | Approx. Output Power Watts |
| 6DQ5 | 24 | 3.2 | 10.5k | 2.5 | 23 | 0.5 | 11 | C | 400 | 200 | -40 | 100 | 12 | 1.5 | 0.1 | 25 |
| 6DQ6B | 18 | 3.6 | 7.3k | 1.2 | 15 | 0.5 | 7 | C | 400 | 200 | -40 | 100 | 12 | 1.5 | 0.1 | 25 |
| 6FH6 | 17 | 3.6 | 16k | 1.2 | 33 | 0.4 | 8 | C | 400 | 200 | -40 | 100 | 12 | 1.5 | 0.1 | 25 |
| 6GC6 | 17.5 | 4.5 | 6.6k | 1.2 | 15 | 0.55 | 7 | C | 400 | 200 | -40 | 100 | 12 | 1.5 | 0.1 | 25 |
| 6GJ5 | 17.5 | 3.5 | 7.1k | 1.2 | 15 | 0.26 | 6.5 | C | 500 | 200 | -75 | 180 | 15 | 5 | 0.43 | 63 |
| 6GJ5 | 17.5 | 3.5 | 7.1k | 1.2 | 15 | 0.26 | 6.5 | AB1 | 500 | 200 | -43 | 85 | 4 |  |  | 35 |
| 6HF5 | 28 | 5.5 | 11.3k | 2.25 | 24 | 0.56 | 10 | C | 500 | 140 | -85 | 232 | 12.5 | 8 | 0.76 | 77 |
| 6HF5 | 28 | 5.5 | 11.3k | 2.25 | 24 | 0.56 | 10 | AB1 | 500 | 140 | -46 | 133 | 4.5 |  |  | 58 |
| 6JB6 | 17.5 | 3.5 | 7.1k | 1.2 | 15 | 0.2 | 6 | C | 500 | 200 | -75 | 180 | 13.3 | 5 | 0.43 | 63 |
| 6JB6 | 17.5 | 3.5 | 7.1k | 1.2 | 15 | 0.2 | 6 | AB1 | 500 | 200 | -42 | 85 | 4.2 |  |  | 35 |
| 6JE6 | 30 | 5 | 10.5k | 2.5 | 24.3 |  | 14.5 | C | 500 | 125 | -85 | 222 | 17 | 8 | 0.82 | 76 |
| 6JE6 | 30 | 5 | 10.5k | 2.5 | 24.3 |  | 14.5 | AB1 | 500 | 125 | -44 | 110 | 3.9 |  |  | 47 |
| 6JG6A | 17 | 3.5 | 10k | 1.6 | 22 | 0.7 | 9 | C | 450 | 150 | -80 | 202 | 20 | 8 | 0.75 | 63 |
| 6JG6A | 17 | 3.5 | 10k | 1.6 | 22 | 0.7 | 9 | AB1 | 450 | 150 | -35 | 98 | 4.5 |  |  | 38 |
| 6JM6 | 17.5 | 3.5 | 7.3k | 1.2 | 16 | 0.6 | 7 | C | 500 | 200 | -75 | 190 | 13.7 | 4 | 0.32 | 61 |
| 6JM6 | 17.5 | 3.5 | 7.3k | 1.2 | 16 | 0.6 | 7 | AB1 | 500 | 200 | -42 | 85 | 4.4 |  |  | 37 |
| 6JN6 | 17.5 | 3.5 | 7.3k | 1.2 | 16 | 0.34 | 7 |  |  |  |  |  |  |  |  |  |
| 6JS6C | 30 | 5.5 |  | 2.25 | 24 | 0.7 | 10 |  |  |  |  |  |  |  |  |  |
| 6KD6 | 33 | 5 | 14k | 2.85 | 40 | 0.8 | 16 | GG | 800 | 0 | -11 | 150 |  |  | 12.5 | 82 |
| 6LB6 | 30 | 5 | 13.4k | 2.25 | 33 | 0.4 | 18 |  |  |  |  |  |  |  |  |  |
| 6LG6 | 28 | 5 | 11.5k | 2 | 25 | 0.8 | 13 |  |  |  |  |  |  |  |  |  |
| 6LQ6 | 30 | 5 | 9.6k | 2.5 | 22 | 0.46 | 11 |  |  |  |  |  |  |  |  |  |
| 6MH6 | 38.5 | 7 | 14k | 2.65 | 40 | 1.0 | 20 |  |  |  |  |  |  |  |  |  |